- Headline: "India's Premium Mica — Muscovite, Phlogopite & Synthetic Grades for Cosmetics, Electricals & Industry — Shipped Worldwide"

Sub-headline: wet ground, dry ground, sheet, splittings, flakes — custom mesh and coating available

- Real imagery: mica flake close-up, grinding plant, packed containers — not generic stock photos

- Product grades catalogue

| Product | Type | Key specs | Primary use |
| --- | --- | --- | --- |
| Wet ground mica | Muscovite | D50: 10–150µm, aspect ratio 40:1+, whiteness 85–92, moisture <0.5% | Cosmetics, paint, plastics filler |
| Dry ground mica | Muscovite / Phlogopite | Mesh: 60–325, brightness 78–88, SiO₂ %, Al₂O₃ %, Fe₂O₃ max | Joint compound, roofing, rubber, wallboard |
| Coated mica (TiO₂/Fe₂O₃) | Muscovite | Particle size 5–200µm, pearlescent effect, colour range (silver, gold, bronze), REACH cert | Cosmetics (eyeshadow, lipstick, highlighter), automotive paint |
| Sheet mica | Muscovite / Phlogopite | V-1 to V-4 grade, thickness 0.1–3mm, dielectric strength, heat resistance to 500°C / 1000°C | Electrical insulation, heating elements, capacitor plates |
| Mica splittings | Muscovite / Phlogopite | Size: 1"×1" to 4"×4", thickness <0.1mm, clear / ruby / green grades | Mica paper, reconstituted mica boards, commutator mica |
| Mica powder (cosmetic) | Muscovite | D50 15–35µm, whiteness 90+, heavy metals per EU 1223/2009, no child-labour declaration | Makeup, skincare, soap, candle making |
| Phlogopite mica | Phlogopite | Heat stability to 1000°C, MgO content, flexibility, low Al₂O₃ | High-temp insulation, welding rods, foundry |

- Custom mesh size, coating type and packaging available on request

- Downloadable TDS for each product

- Origin, geology & sourcing map

India supplies ~60% of the world's sheet mica and is a dominant force in ground mica — this section shows buyers you are a knowledgeable insider, not a trading desk

- Jharkhand (Koderma, Giridih, Hazaribagh): world's finest muscovite sheet mica — ruby, green, clear grades; V-1 quality available

- Andhra Pradesh (Nellore district): large-scale muscovite, major source for ground mica and splittings export

- Rajasthan (Bhilwara, Ajmer, Tonk): phlogopite and muscovite; growing processing hub with modern grinding units

- Brief geology: muscovite forms in pegmatites (Al-rich, K-rich environments); phlogopite in Mg-rich metamorphic zones — this determines chemical properties and heat resistance

- Processing chain: mine → hand cobbing → sizing → sheet grading OR crushing → grinding (dry/wet) → classification → surface treatment → packaging

- CTA for local mines and processors: "Are you a mica mine, cobbing unit or grinding processor in Jharkhand, AP or Rajasthan? Partner with us for consistent export offtake"

- Industry applications

Mica has more diverse end-use industries than almost any other mineral — a cosmetics buyer and an electrical engineer need to hear completely different things. Address each vertical separately.

- Cosmetics & personal care (EU/USA/Japan): wet ground muscovite and TiO₂-coated pearlescent mica for eyeshadow, foundation, blush, lip gloss, skincare; key concerns — heavy metals (EU 1223/2009 limits for Pb, As, Hg, Cd), particle size for skin feel, ethical sourcing declaration, REACH compliance

- Paints & coatings: dry/wet ground mica as functional extender — improves barrier properties, reduces cracking, enhances sheen; high aspect ratio critical; used in automotive primers, industrial coatings, marine paints

- Electrical insulation: sheet mica and splittings for commutator mica, slot wedges, heating element supports, capacitor plates; phlogopite preferred for high-temp (500–1000°C) applications; dielectric strength and flexibility the key parameters

- Electronics & EV batteries: mica tape for cable wrapping, thermal management sheets, fire-resistant barriers in lithium-ion battery packs — fast-growing demand from EV sector

- Plastics & rubber: dry ground mica as reinforcing filler — improves stiffness, reduces warpage in injection-moulded nylon/PP; reduces creep in HDPE pipes

- Construction: mica in joint compounds (drywall finishing), acoustic ceiling tiles, roofing felt — US and European construction sector is a large buyer of dry ground mica

- Welding electrodes & foundry: phlogopite mica powder as flux coating binder; improves slag fluidity and arc stability

- Candle & soap making (craft / D2C segment): cosmetic-grade mica powder — small quantity buyers but high volume online (Amazon, Etsy suppliers) — mention minimum order for this segment

# 5. Types of Mica We Export

### 1. Muscovite Mica

- Most widely used

- Excellent electrical insulation

- High dielectric strength

Applications:

- Electrical insulation

- Paints & coatings

- Plastics & rubber

### 2. Phlogopite Mica

- Better thermal resistance than muscovite

- Suitable for high-temperature applications

Applications:

- Foundries

- Welding electrodes

- Heat insulation systems

### 3. Mica Forms Available

- Mica flakes (coarse / fine)

- Mica powder (200 mesh / 325 mesh / customized)

- Mica sheets / blocks

# FAQ

### 1. What is the difference between muscovite and phlogopite mica?

Muscovite offers better electrical insulation, while phlogopite performs better in high-temperature applications.

### 2. What mesh sizes can you supply?

We can supply mica powder from 100 mesh up to 500 mesh, including custom sizes based on your requirement.

### 3. Do you provide samples before bulk orders?

Yes, samples can be arranged for quality evaluation before confirming bulk shipments.

### 4. What industries use mica?

Mica is widely used in paints, plastics, rubber, electronics, cosmetics, and construction industries.

### 5. What is the minimum order quantity?

Typically, 1 x 20 FT container, but smaller trial quantities can be discussed.

### 6. Can you supply consistent quality for repeat orders?

Yes, we work with selected suppliers to maintain consistency across shipments.

### 7. How is Indian mica different from other origins?

Indian mica is known for its natural purity, cost competitiveness, and strong processing ecosystem.